# Remote microphone virtual sensing with multi-microphone configurations

Achilles Kappis1,*, Jordan Cheer1 , Jihui (Aimee) Zhang1

1Institute of Sound and Vibration Research, University of Southampton, University Road, Southampton, SO17 1BJ, United Kingdom

*A.Kappis@soton.ac.uk

### ABSTRACT

Local active noise control applications can greatly benefit from virtual sensing when error microphones cannot be positioned at the location where control is required. A previous study has provided evidence that monitoring microphone configurations comprising nested sub-arrays have the potential to enhance sound field estimation performance in diffuse sound fields. This work extends these findings by investigating the performance of similar microphone configurations in sound fields generated by a two-dimensional source configuration. The estimation accuracy of linear and circular monitoring microphone setups is evaluated, and the findings suggest that the use of nested arrays provides higher estimation accuracy at a larger spatial extent. Furthermore, the robustness of the microphone setups against perturbations is investigated, and it is shown that the configurations with nested arrays suffer from increased sensitivity, which can be partially alleviated with regularisation.

# Εκτίμηση ηχητικών πεδίων με συστοιχίες ένθετων μικροφώνων

### ΠΕΡΙΛΗΨΗ

Πολλές εφαργμογές ενεργούς ηχομείωσης μπορούν να επωφεληθούν από την τηλεσκοπική ανίχνευση ηχητικών πεδίων, ειδικά όταν η τοποθέτηση μικροφώνων στην θέση που απαιτείται έλεγχος δεν είναι εφικτή. Προηγούμενη μελέτη έδειξε  ότι οι διατάξεις μικροφώνων με ένθετες υπο-συστοιχίες μπορούν να βελτιώσουν την εκτίμηση διάχυτων ηχητικών πεδίων. Η παρούσα εργασία επεκτείνει την προηγούμενη έρευνα διερευνώντας την χρήση παρόμοιων διατάξεων για την ανίχνευση ηχητικών πεδίων παραγόμενα από δισδιάστατες συστοιχίες πηγών. Για την ανίχνευση χρησιμοποιούνται γραμμικές και κυκλικές διατάξεις μικροφώνων και τα αποτελέσματα δείχνουν ότι οι διατάξεις αποτελούμενες από υπο-συστοιχίες παρέχουν υψηλότερη ακρίβεια εκτίμησης σε μεγαλύτερη χωρική έκταση. Επιπλέον, διερευνάται η ευαισθησία των διατάξεων σε σφάλματα δείχνοντας ότι η χρήση υπο-συστοιχιών αυξάνει την ευαισθησία στον θόρυβο, κάτι που μπορεί να αντιμετωπιστεί μερικώς με την χρήση κανονικοποίησης.

## Introduction

Local active control applications provide feasible solutions to noise control problems [1]. However, in many applications, such as the active headrest [2], it is impractical or even impossible to position microphones at the control location. The augmentation of local active control systems with Virtual Sensing (VS) has led to a plethora of proposed solutions [3]. Information of the sound field at positions remote from the monitoring microphones provided by the VS part of the system is used, to minimize the disturbance at these positions resulting in a shift of the control point to the desired location. Providing accurate sound field estimation is crucial for the control process, and poor performance of the VS algorithm can significantly limit the overall performance of the system [4].

Conventional active control algorithms utilise pressure microphones to sample the sound field. However, past studies have shown that the use of pressure gradient information can enhance both estimation [5] and control performance [6, 7]. Additionally, it has been demonstrated that using pressure measurements of closely spaced sensors provides similar results to using a pressure and pressure gradient component when the distance between the sensors is significantly shorter than the wavelength of the primary disturbance [6, 7].

Building upon a previous study [8], the current work aims to investigate the sound field estimation performance of nested-array microphone configurations in simulated random sound fields. A linear and circular array with their elements formed by subsets of microphones of a four-element sub-array are used for the evaluation.

## Problem formulation

The block diagram of the system used in this work is shown in Figure 1.1. A sound field generated by Nv sources with complex source strengths v=v1, v2, v3…,vNvT, where  · T denotes transposition. The signals are assumed to be generated by wide-sense stationary random processes, characterised by their power spectral density. The disturbance field at Ne virtual microphones, de=de1, de2, de3…,dNeT, and at Nm monitoring microphones, dm=dm1, dm2, dm3…,dNmT, are given by

de=Pev				(1.1)

dm=Pmv,				(1.2)

where Pe∈CNe×Nv and Pm∈CNm×Nv, are the frequency response matrices from the sources to the virtual and monitoring microphones respectively. The disturbance at the virtual microphones, de, is estimated by filtering the measured signals dm with the observation filter O, where the symbol  ·  denotes an estimated quantity. The estimation error is given by the difference of the true and estimated sound field as

e=de- de=de- Odm.			(1.3)

Figure 1.1: Block diagram of the virtual sensing estimation system. To estimate the disturbance signals de, the observation filter O is applied to the measured signals dm to give the estimated disturbance de.

The optimal, in the least-squares sense, observation filter can be calculated by minimising the mean squared error [9]

J=EeHe=traceEeeH=traceSee-SmeOH-OSmeH+OSmmOH (1.4)

where E ·  is the expectation operator and [ · ]H denotes Hermitian transposition. The quantities See and Smm are the power spectral densities of the disturbance field at the virtual and monitoring microphones respectively and Sme is the cross-spectral density between the monitoring and virtual microphone signals given by

See=EdedeH=PeEvvHPeH=PeSvvPeH		(1.5)

Smm=EdmdmH=PmEvvHPmH=PmSvvPmH		(1.6)

Sme=EdedmH=PeEvvHPmH=PeSvvPmH.		(1.7)

The quantity Svv=EvvH, is the power spectral density of the source strengths.

The solution to the cost function of Equation (1.4) is given by [10]

O=SmeSmm+βI-1= PeSvvPmHPmSvvPmH+βI-1,		(1.8)

where Equation (1.6) and Equation (1.7) were used to expand the cross- and power spectral density terms. The regularization term β is a non-negative real scalar and I is an Nm×Nm identity matrix. The regularisation term βI is used to constrain the magnitude of the filter terms and corresponds to adding noise to the microphone signals [10] which can be frequency dependent. At the expense of reduced estimation accuracy [4], regularised filters show increased robustness to perturbations in the monitoring microphone responses [11].

The performance metric used in this work is the normalised mean squared estimation error, given by

Le=10log10traceSεεtraceSee,			(1.9)

with Sεε=EeeH being the mean squared estimation error. The metric acquires

negative values for increased estimation performance with the perfect estimation accuracy being represented by -∞.

## System geometry

To facilitate comparison with previous studies [8], a uniform linear array (ULA) with inter-element distance lULA=0.15 m and a uniform circular (UCA) array with radius lUCA=0.4 m were deployed. The elements of the arrays are formed by nested sub-arrays with four microphones arranged in two orthogonal pairs, with the distance between the microphones of each pair equal to lp=0.05 m. Different configurations were implemented by using subsets of the sub-arrays, resulting in four distinct monitoring microphone arrangements shown in Figure 2.1. Two of the arrangements are formed by using one pair from the sub-arrays, parallel to the y axis for the ULAy and aligned radially for the UCAr. The other two are implemented by adding a second pair in each sub-array, to form ULAxy and UCAr. Finally, arrays with a single pressure sensor at each element position are deployed to allow comparison with conventional monitoring setups. The monitoring setups are centred at the origin with all receivers being modeled as perfect omnidirectional pressure transducers and scattering effects are not considered.

The microphone configurations are immersed in a sound field generated by thirty two ideal point sources equally spaced on a circle centred at the origin with a radius of 3 m. The sources are assumed to be uncorrelated and of equal strength resulting Svv=vI, with v the source strength, which in the current work is set to unity. The sound field is estimated on a square 3 m×3 m grid of 85×85 uniformly distributed virtual microphones. The source arrangement is shown in Figure 2.1, where the element positions of the ULA and UCA and the area where the virtual microphones are placed are also shown.

## Simulations

#### 3.1 Estimation performance over space

The estimation performance of the deployed arrays over the grid of virtual microphones is evaluated at the frequencies of 250 Hz, 500 Hz and 1 kHz. During the filter calculations, a frequency-independent regularization factor β=10-3 was applied to minimise numerical errors introduced by the inversion of the power spectral density matrix Smm. The effect of the condition of the matrix is investigated in Section 3.2 in more depth.

The term estimation zone is used in this work to denote the area over which the normalized estimation error is at most -10 dB. The estimation zones for the deployed monitoring microphone configurations are illustrated in Figure 3.1 as the areas enclosed by the lines. A trend common to all microphone configurations is the extension of the estimation zone along the axis of microphones when the wavelength of the excitation frequency is significantly larger than the distance between them. The dependence of the size of the estimation zone on the distance between the microphones and the wavelength is evident at higher frequencies. When the wavelength is comparable to or shorter than the distance between the array element

| - ULAs. | - UCAs. | - Complete system. |
| --- | --- | --- |
| Figure 2.1: The geometry of the implemented system. The three (a) ULA and (b) UCA configurations are shown where the pairs of the pairs of the sub-arrays are color-coded. The fully populated configurations are formed by the union of the two pairs of the sub-arrays (red and yellow sensors). The complete system is shown in (c) where the green area represents the grid of virtual microphones. |  |  |

positions the shape of the estimation zones becomes that acquired by the sub-arrays alone. Similar findings were reported in past studies [6, 7, 8] and reinforce the suggestion that the estimation performance depends on the coherence between the monitoring and virtual microphones [11].

An extension in the estimation zones is consistently exhibited along the axes of microphone pairs. The effect is prominent for both the ULA and UCA comprising two-element sub-arrays, where the extension of the estimation zone along the axis of the sub-array pairs is evident. Similarly, the linear setups with two microphone sub-arrays and single microphone as elements exhibit comparable estimation zone extension in the x-axis as they have their structure along this direction is the same. Furthermore, the “gaps” in the estimation zones acquired with the fully populated configuration between the configuration’s element positions appear because there is no microphone pair with an axis parallel to these positions. Finally, it is worth noting that increasing the number of microphones for a given array topology monotonically increases the estimation performance, as measured by the extent of the estimation zones.

#### 3.2 Sensitivity to perturbations

It has been demonstrated that dense microphone setups suffer from increased sensitivity to perturbations and noise [4, 8]. The robustness of the deployed configurations is explored here via the condition number of the monitoring microphone power spectral matrix κ(Smm). High condition number values suggest that the matrix is ill-conditioned. The regularisation term βI is introduced to control the conditioning of the matrix and increase robustness [4]. However, the appropriate regularisation factor has a strong dependence on the values of the Smm matrix. Since regularising is equivalent to the introduction of noise in the measured responses [10], in this work, the value of the regularisation factor β is chosen such that it will provide a specified Signal-to-Noise Ratio (SNR) in the monitoring microphone measured

| - ULAs f=250 Hz. | - ULAs f=500 Hz. | - ULAs f=1 kHz. |
| --- | --- | --- |
| - UCAs f=250 Hz. | - UCAs f=500 Hz. | - UCAs f=1 kHz |
| Figure 3.1: Estimation zones for the six deployed arrays at the frequencies of 250 Hz, 500 Hz and 1 kHz. The positions of the array elements are shown as black dots. |  |  |

responses. The SNR is calculated as [11]

β=e2SmmFM2,				(3.1)

where e denotes the normalised Root-Mean-Square (RMS) error due to noise,  · F is the Frobenius norm, M is the number of microphones and the frequency dependence of the value has been suppressed for notational convenience. The error e takes values in the range 0≤e≤+∞ and the SNR can then be calculated in dB as

SNR=10log10e-1=-10log10e			(3.2)

To investigate the sensitivity of the monitoring setups and the effect of regularisation, the condition number and the normalised estimation error, averaged over the virtual microphone grid, for all deployed configurations is calculated for the frequency of 500 Hz and SNR values ranging from 30 dB to -10 dB and are shown in Figure 3.2.

As is evident from the results in the figure, dense microphone configurations, shown in red and yellow lines, exhibit large condition numbers indicating high sensitivity to perturbations. It is worth noting that the linear setups are more sensitive than their circular counterparts, due to the proximity of the array element positions, indicating that the condition of the array is dependent on both the form of the array,

| - ULA configurations. | - UCA configurations. |
| --- | --- |
| Figure 3.2: Condition number of the monitoring microphone power spectral density matrix and the spatially averaged normalised estimation error. The solid lines correspond to the condition number and the dashed lines to the error. |  |

as well as the structure of the sub-array elements.

The condition number exhibits a plateau at high SNR values with saturation in the estimation performance following the same trend. The average estimation performance for the arrays with single microphone elements is similar, despite the difference in the spatial distribution of the error between them as shown Figure 3.1. Between the two microphone sub-array configurations, the linear setup achieves lower estimation error on average. The smaller distance between array positions allows the combination of information as described in the literature [6-8], which is not the case for the circular setup as can be seen in Figure 3.1.e.

In the case of the configurations with fully populated sub-arrays, the circular setup outperforms the linear arrangement. The extended area covered by the circular setup in combination with the considerable increase in the coherence between the monitoring and virtual microphones on the microphone axes, provided by the dense sub-arrays, result in a significant enhancement in the estimation performance.

Finally, the condition of all microphone configurations seems to converge to a low value when the SNR deteriorates, with significant reduction in the average estimation performance.

## Summary

The estimation performance of microphone configurations formed by nested sub-arrays has been evaluated in a simulated sound field generated by a circular array of ideal point sources. The area where the estimation error is less than -10 dB, shows considerable extension along the axis of microphone pairs when the distance between the microphones is significantly shorter than the wavelength of the exciting sound field, supporting findings of past studies [6, 8].

Dense microphone setups achieve lower estimation errors than the conventional arrays but are shown to suffer from increased sensitivity to perturbations. Regularisation can be introduced to increase the robustness of the configurations at the expense of estimation performance. The performance and sensitivity of the microphone configurations are two competing factors, and their balance must be taken into account during the design of the monitoring microphone setups.

## Acknowledgements

This work was supported by the project “IN-NOVA: Active reduction of noise transmitted into and from enclosures through encapsulated structures”, which has received funding from the European Union’s Horizon Europe programme under the Marie Skłodowska-Curie grant agreement no. 101073037 and by UK Research and Innovation under the UK government’s Horizon Europe funding guarantee [grant number EP/X027767/1].

## References

[1] S. J. Elliott, P. A. Nelson, “Active Noise Control”, IEEE Signal Processing Magazine 10, pp. 12–35 (1993).

[2] W. Jung, S. J. Elliott, and J. Cheer, “Estimation of the pressure at a listener’s ears in an active headrest system using the remote microphone technique”, The Journal of Acoustical Society of America 143, pp. 2858–2869 (2018).

[3] D. J. Moreau, B. S. Cazzolato, A. C. Zander, and C. Petersen, “A Review of virtual sensing algorithms for active noise control”, Algorithms 1, pp. 69–99 (2008).

[4] J. Zhang, S. J. Elliott, and J. Cheer, “Robust performance of virtual sensing methods for active noise control”, Mechanical Systems and Signal Processing 152 (2021).

[5] B. Xu, S. D. Sommerfeldt, and T. W. Leishman, “Generalised acoustic energy density”, The Journal of the Acoustical Society of America 130, pp. 1370–1380 (2011).

[6] S. J. Elliott and J. Garcia-Bonito, “Active cancellation of pressure and pressure gradient in a diffuse sound field”, Journal of Sound and Vibration 186, pp. 696–704 (1995).

[7] D. J. Moreau, J. Ghan, B. S. Cazzolato, and A. C. Zhander, “Active noise control in a pure tone diffuse sound field using virtual sensing”, The Journal of the Acoustical Society of America 125, pp. 3742–3755 (2009).

[8] A. Kappis, J. Cheer, and Jihui (Aimee) Zhang, “Remote sensing using multi-microphone configurations for local active noise control applications” in Proceedings of the 30th International Congress on Sound and Vibration, (2024).

[9] S. J. Elliott and J. Cheer, “Modeling local active sound control with remote sensors in spatially random pressure fields”, The Journal of the Acoustical Society of America 137, pp. 1936–1946 (2015).

[10] S. J. Elliott, J. Cheer, J. W. Choi, and Y. Kim, “Robustness and regularization of personal audio systems”, IEEE Transactions on Audio, Speech and Language Processing 20, pp. 2123–2133 (2012).

[11] W. Jung, S. J. Elliott, and J. Cheer, “Combining the remote microphone technique with head-tracking for local active sound control”, The Journal of the Acoustical Society of America 142, pp. 298–307 (2017).

[12] P. Zhang, S. Wang, H. Duan, J. Tao, H. Zou, and X. Qiu, “A study on coherence between virtual signal and physical signals in remote acoustic sensing”, The Journal of the Acoustical Society of America 152, pp. 2840–2848 (2022).